DEPARTMENT OF COMPUTER & SOFTWARE ENGINEERING

COLLEGE OF E&ME, NUST, RAWALPINDI

Subject:

Digital Image Processing

Project Report

Real-time Image Analysis for Self Driving Capabilities

SUBMITTED TO:

Mam LE Sundas Ashraf

Dr Asad Khan

SUBMITTED BY:

Student Name

Muhammad Muhtashim     		Reg# 407384

Muhammad Taha				Reg # 471609

Muhammad Sohaib Afzal			Reg# 411928

Muhammad Hashir Ashraf Awan 	Reg# 408557

DE- 44 Department C&SE – Syn B

Submission Date: 30/04/25

Introduction

This report outlines the implementation of a real-time object detection system that combines lane detection, obstacle recognition, and directional decision-making to process video streams effectively. This system is capable of detecting obstacles in the environment, highlighting lanes on the road, and making intelligent decisions based on road conditions. The core application of this system lies in autonomous vehicle navigation or robotic systems, where understanding road lanes and obstacles in real-time is crucial for safe movement.

Overview

The proposed system is built using OpenCV, a computer vision library, and Numpy, a powerful library for numerical operations. The system works by processing video input from a file and performs the following tasks:

Lane Detection: Highlights the most prominent lane on the road, considering road edges and center alignment.

Obstacle Detection: Detects red-colored obstacles within the frame.

Directional Decision Making: Based on detected obstacles, the system computes possible movement directions for the agent (e.g., forward, right, left, or backward).

Methodology

1. Lane Highlighting

Lane detection is a crucial aspect of driving assistance systems. The lane detection function performs edge detection using the Canny algorithm and applies morphological transformations to clean up the edges. The processed image is then analyzed to detect the largest region representing the lane. The algorithm follows these steps:

Gray Conversion: The frame is converted to grayscale.

Edge Detection: The Canny edge detector identifies boundaries within the frame.

Morphological Operations: These operations close gaps between detected edges to form a more continuous lane.

Component Analysis: The largest lane region is selected based on area, position, and alignment with the center of the frame.

2. Obstacle Detection

The obstacle detection function uses a color-based approach to detect obstacles in the frame. By converting the frame from BGR (Blue-Green-Red) to HSV (Hue-Saturation-Value) color space, we define a range for red-colored objects. The system detects obstacles using the following process:

HSV Conversion: Converts the BGR frame to HSV to isolate red-colored objects.

Mask Creation: Creates a mask that highlights regions in the image matching the defined color range.

Contour Detection: Identifies contours within the masked regions and filters out small or irrelevant contours.

Bounding Boxes: For each valid contour, a bounding rectangle is drawn, marking the obstacle location.

3. Directional Decision Making

The directional decision-making function evaluates the road environment by analyzing the available space in different directions (up, down, left, and right). Based on obstacle detection and lane analysis, the function decides on the most suitable direction for movement. The decision-making process involves:

Checking Forward: Verifying if the space ahead is free of obstacles.

Checking Right, Left, and Backward: Evaluating other directions if the forward space is blocked by obstacles. The system decides based on the availability of a clear path in any direction.

Updating Coordinates: If a clear path is found, the system updates the coordinates of the moving object to simulate movement.

4. Real-Time Video Processing

The video processing function integrates the lane highlighting, obstacle detection, and directional decision-making processes. The system processes each frame of the video and applies the relevant operations sequentially, displaying the results in real time. It follows these steps:

Video Input: Captures the video stream from the specified file.

Frame Processing: Each frame is processed to detect lanes and obstacles.

Decision Update: Based on the detected obstacles, the system updates the movement direction.

Real-Time Display: Displays the processed video frames in a window, showing the highlighted lanes and detected obstacles.

5. Code Implementation

Here is an outline of the core functions and their roles:

import cv2

import numpy as np

# === Obstacle Detection Function ===

def detect_obstacles(frame):

hsv_frame = cv2.cvtColor(frame, cv2.COLOR_BGR2HSV)

# Define HSV range for obstacle color (tuned for red - adjust as needed)

color_range_min = np.array([0, 45, 50])

color_range_max = np.array([15, 255, 255])

obstacle_mask = cv2.inRange(hsv_frame, color_range_min, color_range_max)

cv2.imshow('Detected Obstacles Mask', obstacle_mask)

contours, _ = cv2.findContours(obstacle_mask, cv2.RETR_TREE, cv2.CHAIN_APPROX_SIMPLE)

detected_boxes = []

for cnt in contours:

if cv2.contourArea(cnt) > 150:

x, y, width, height = cv2.boundingRect(cnt)

detected_boxes.append((x, y, width, height))

cv2.rectangle(frame, (x, y), (x + width, y + height), (0, 0, 255), 2)

cv2.putText(frame, "Object", (x, y - 8),

cv2.FONT_HERSHEY_SIMPLEX, 0.5, (0, 0, 255), 2)

return frame, detected_boxes

def directional_decision_making(road,object,x1,x2, y1,y2):

road[x1:x2, y1:y2] = object

check_upward=True

check_right=True

check_left=True

check_backward=True

for i in range (y1,y2):                                                             ### check for forward

if [road[x1-1,i][0],road[x1-1,i][1],road[x1-1,i][2]]==[0,255,255]:        ### move upwards

pass

else:

check_upward=False

if (check_upward==True):

x1-=1

x2-=1

else:

for i in range (x1,x2):                                                             ### check for right

if [road[i,y2+1][0],road[i,y2+1][1],road[i,y2+1][2]]==[0,255,255] or [road[i,y2-1][0],road[i,y2-1][1],road[i,y2-1][2]]==[0,0,255]:        ### move right

pass

else:

check_right=False

if (check_right==True):

y1+=1

y2+=1

else:

for i in range (x1,x2):                                                             ### check for left

if [road[i,y1-1][0],road[i,y1-1][1],road[i,y1-1][2]]==[0,255,255] or [road[i,y2+1][0],road[i,y2+1][1],road[i,y2+1][2]]==[0,0,255]:        ### move left

pass

else:

check_left=False

if (check_left==True):

y1-=1

y2-=1

else:

for i in range (y1,y2):                                                             ### check for backward

if [road[x1+1,i][0],road[x1+1,i][1],road[x1+1,i][2]]==[0,255,255]:        ### move backward

pass

else:

check_backward=False

if (check_backward==True):

x1+=1

x2+=1

return road,x1,x2, y1,y2

# === Lane Highlighting Function ===

def highlight_largest_lane_region(frame, edge_pixel_thresh=100):

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

edges = cv2.Canny(gray, 50, 150)

kernel = np.ones((5, 5), np.uint8)

closed_edges = cv2.morphologyEx(edges, cv2.MORPH_CLOSE, kernel)

inv = cv2.bitwise_not(closed_edges)

height, width = inv.shape

bottom = inv[height // 2:, :]

_, binary = cv2.threshold(bottom, 127, 255, cv2.THRESH_BINARY)

num_labels, labels, stats, centroids = cv2.connectedComponentsWithStats(binary, connectivity=8)

min_area = 4000

best_label = -1

best_score = 0

for i in range(1, num_labels):

x, y, w, h, area = stats[i]

if area < min_area:

continue

component_mask = (labels == i).astype(np.uint8) * 255

edge_pixels_in_component = cv2.bitwise_and(bottom, bottom, mask=component_mask)

nonzero_edge_count = cv2.countNonZero(edge_pixels_in_component)

if nonzero_edge_count < edge_pixel_thresh:

continue

cx, cy = centroids[i]

distance_from_center = abs((width / 2) - cx)

vertical_position_score = h

score = area - 5 * distance_from_center + vertical_position_score

if score > best_score:

best_score = score

best_label = i

if best_label == -1:

return frame, edges

mask = np.zeros_like(binary)

mask[labels == best_label] = 255

full_mask = np.zeros_like(gray)

full_mask[height // 2:, :] = mask

contours, _ = cv2.findContours(full_mask, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE)

overlay = frame.copy()

for cnt in contours:

cv2.drawContours(overlay, [cnt], -1, (0, 255, 255), -1)  # filled yellow

blended = cv2.addWeighted(overlay, 0.4, frame, 0.6, 0)

return overlay, edges

# === Main Video Processor ===

def process_video_realtime(video_path):

cap = cv2.VideoCapture(video_path)

obj = np.ones((20, 20, 3), dtype=np.uint8) * np.array([0, 0, 255], dtype=np.uint8)

x1, x2 = 400, 420

y1, y2 = 250 - 10, 250 + 10  # center of 500x500 frame

if not cap.isOpened():

print("Failed to open video.")

return

while True:

ret, frame = cap.read()

frame = cv2.resize(frame, (640, 480))

if not ret:

break

# Highlight lane

frame, edge_map = highlight_largest_lane_region(frame)

# Detect obstacles and draw them on lane frame

final_frame, _ = detect_obstacles(frame)

final_frame, x1, x2, y1, y2 = directional_decision_making(final_frame, obj, x1, x2, y1, y2)

# Show results

cv2.imshow("Edge Map", edge_map)

cv2.imshow("Lane and Obstacle Detection", final_frame)

if cv2.waitKey(1) & 0xFF == ord('q'):

break

cap.release()

cv2.destroyAllWindows()

# === Run the Combined Pipeline ===

process_video_realtime('E:/Documents/6th Semester/DIP/Lab/Project/DIP Project Videos/q1.mp4')

6. Real-Time Processing Flow

The main video processing function coordinates the system's operations:

def process_video_realtime(video_path):

cap = cv2.VideoCapture(video_path)

obj = np.ones((20, 20, 3), dtype=np.uint8) * np.array([0, 0, 255], dtype=np.uint8)

x1, x2 = 400, 420

y1, y2 = 250 - 10, 250 + 10  # center of 500x500 frame

if not cap.isOpened():

print("Failed to open video.")

return

while True:

ret, frame = cap.read()

frame = cv2.resize(frame, (640, 480))

if not ret:

break

# Highlight lane

frame, edge_map = highlight_largest_lane_region(frame)

# Detect obstacles and draw them on lane frame

final_frame, _ = detect_obstacles(frame)

final_frame, x1, x2, y1, y2 = directional_decision_making(final_frame, obj, x1, x2, y1, y2)

# Show results

cv2.imshow("Edge Map", edge_map)

cv2.imshow("Lane and Obstacle Detection", final_frame)

if cv2.waitKey(1) & 0xFF == ord('q'):

break

cap.release()

cv2.destroyAllWindows()

- Flow chart

Results and Evaluation

The system successfully identifies lanes on the road and detects obstacles using real-time video feeds. Lane highlighting helps track the central road region, while obstacle detection identifies red-colored objects in the environment. Directional decision-making allows the system to compute potential movement options based on the detected obstacles, simulating real-time navigation.

Lane Detection: The lane detection algorithm efficiently identifies lane boundaries, even under partial occlusions.

Obstacle Detection: The obstacle detection works effectively for red-colored objects, marking obstacles with bounding boxes.

Directional Decision Making: The system can navigate around obstacles by evaluating available movement directions.

Output:

Pre-recorded video check:

https://drive.google.com/drive/folders/1utqxvC7IJvs9J0k3RKJoTLrGlA9ayWy7?usp=drive_link

Conclusion

The real-time object detection system, equipped with lane highlighting and directional decision-making capabilities, proves to be a reliable tool for autonomous navigation. This framework can be adapted to different environments and enhanced further by incorporating more sophisticated obstacle recognition and motion prediction algorithms. Future improvements can include the detection of multiple colors for broader obstacle detection and the use of machine learning for better decision-making accuracy.